# Joint Optimization of Energy, Latency and Age of

# Information for MEC Task Offloading Using

# TLBO-HHO Algorithm

1st  Given Name Surname

dept. name of organization (of Aff.)

name of organization (of Aff.)

City, Country

email address or ORCID

2nd  Given Name Surname

dept. name of organization (of Aff.)

name of organization (of Aff.)

City, Country

email address or ORCID

3rd  Given Name Surname

dept. name of organization (of Aff.)

name of organization (of Aff.)

City, Country

email address or ORCID

Abstract—Mobile  Edge  Computing  (MEC)  provides  low-  latency  and  energy-efficient  computing  services  for  resource-  constrained mobile devices by deploying computing resources  at  the  network  edge.  However,  in  multi-user  scenarios,  how  to effectively make task offloading decisions while optimizing  system energy consumption, latency, and information freshness  remains a challenging problem. This paper proposes a hybrid  metaheuristic algorithm (TLBO-HHO) that integrates Teaching-  Learning-Based Optimization (TLBO) and Harris Hawks Op-  timization  (HHO)  to  solve  the  task  offloading  and  resource  allocation optimization problem in MEC environments consid-  ering Age of Information (AoI). The main contributions include:

(1) establishing a tri-objective optimization model considering  energy consumption, latency, and AoI, using  queuing theory  to analyze task waiting time, and proving its NP-hardness; (2)  designing a two-phase integrated TLBO-HHO fusion algorithm  that balances global exploration and local exploitation through  an adaptive switching mechanism; (3) experimental results show  that compared to the original TLBO, GA, and GWO algorithms,  TLBO-HHO improves fitness values by 65.2%, 85.4%, and 74.6%  respectively, while effectively reducing information staleness.

Index  Terms—Mobile  edge  computing,  task  offloading,  re-  source  allocation,  age  of information,  teaching-learning-based  optimization, Harris hawks optimization, two-phase integration.

I.  INTRODUCTION

With  the  rapid  development  of  Internet  of  Things  (IoT) and 5G/6G communication technologies, the amount of data generated by mobile devices is growing exponentially. Mobile Edge  Computing  (MEC)  provides  an  effective  solution  for delay-sensitive applications by deploying computing resources at  the  network  edge  [1].  In  MEC  systems,  task  offloading decisions  directly  affect  system  performance   and  need  to balance between  energy  consumption  and latency. In recent years,  with  the  increasing  demand  for  real-time  communi- cation  and  real-time  decision-making,  Age  of  Information (AoI) has become an important metric for measuring system information freshness [15], and incorporating AoI optimization has become a research trend.

Existing research methods include convex optimization [2], game  theory,  deep  reinforcement  learning  [3],  and  meta- heuristic algorithms. However, single metaheuristic algorithms

have  inherent  defects:  Genetic  Algorithm  (GA)  is  prone  to premature convergence, and Grey Wolf Optimizer (GWO) has slow convergence speed [4]. Hybrid metaheuristic algorithms can effectively overcome these limitations by combining the advantages of different algorithms.

The main contributions of this paper are as follows:

•   We establish a tri-objective optimization model consider- ing energy consumption, latency, and AoI, with weight settings  of  0.25-0.35-0.40,   analyze  task  waiting  time through queuing theory, and prove its NP-hard nature.

•   We propose  a two-phase integrated TLBO-HHO fusion algorithm that achieves dynamic balance between explo- ration  and  exploitation  through  an  adaptive  probability mechanism (0.7→ 0.1).

•   We validate the effectiveness of the  algorithm in infor- mation timeliness optimization in a scenario containing

20 devices, 4 edge servers, 2 cloud servers, and 40 tasks. II.  RELATED WORK

A. Metaheuristic Optimization Algorithms

Rao  [5] proposed the  TLBO  algorithm that  simulates the teaching process, which has the  advantages of few parame- ters  and  stable convergence. Heidari et  al.  [6]  proposed the HHO algorithm that simulates Harris hawks’ hunting behav- ior,  achieving  a  success  rate  of  85-95%  when  dealing  with multimodal functions. In recent research, Chen et al. [7] and

Tu et al. [8] respectively proposed hybrid improved versions of HHO, but the application of TLBO-HHO in MEC is still scarce.

B. MEC Task Offloading Optimization

Task offloading optimization methods can be divided into three categories:

(1) Mathematical optimization methods: Wang et al. [9] pro- posed a joint computation offloading and interference manage- ment scheme based on Lagrangian dual decomposition, which theoretically guarantees the optimality of the solution. Liu et al.  [10]  designed  a  delay-optimal task  scheduling  algorithm based on Lyapunov optimization, which can minimize average

delay while ensuring queue stability. These methods have solid theoretical foundations but are difficult to handle non-convex problems.

(2) Machine learning methods: Wang et al.  [11] proposed a dynamic trajectory control scheme based on deep reinforce- ment learning, achieving adaptive resource allocation in UAV- assisted MEC. Zang et al. [13] designed a federated deep re- inforcement learning framework that optimizes task offloading decisions while protecting user privacy. These methods have strong adaptability but require large amounts of training data.

(3) Metaheuristic methods: Li and Zhu [12] applied genetic algorithms to optimize task offloading in MEC, achieving a 30% reduction in energy consumption. Ali et al. [14] proposed a multi-objective HHO algorithm for task scheduling in cloud- fog  computing,  which  performs  well  in  handling  NP-hard problems and can achieve up to 56% energy reduction.

C. Age of Information (AoI) Optimization

AoI, as a key metric for measuring information freshness, was  first  proposed  by  Kaul  et  al.  [16]  in  2012  to  quantify the timeliness of information at the receiver. In IoT and MEC scenarios, the importance  of AoI is increasingly prominent. Abd-Elmagid  et  al.  [15]  comprehensively  reviewed the role of AoI in the Internet of Things, pointing out that optimizing AoI in status update systems is crucial for real-time decision- making.

Liu et al. [17] studied the AoI-aware task computing prob- lem in vehicular edge computing, proposing an asynchronous deep reinforcement learning method that can simultaneously optimize computing resource allocation and AoI performance. Zhou  et  al.  [18]  designed  a  contract  matching  mechanism considering AoI, achieving efficient allocation of computing resources in vehicular fog computing environments. However, existing  research  rarely  combines  AoI  with  metaheuristic algorithms  for  MEC  task  offloading  optimization,  which  is one of the innovations of this paper.

III.  SYSTEM MODEL AND PROBLEM STATEMENT

A. Network Architecture and Scenario Description

We consider a typical three-layer MEC network architec- ture, as shown in Fig. 1. This architecture reflects the hierarchi- cal distribution of computing resources in actual deployment, consistent with the deployment patterns of commercial MEC platforms (such as AWS Wavelength and Azure Edge Zones).

The system contains the following components:

•   Device  layer:  D  =  {1, 2,..., D} represents  D   =  20 mobile devices, including low-end IoT devices (40%, 0.2- 0.5 GHz), mid-range mobile devices (40%, 0.8-1.5 GHz), and high-end devices (20%, 2.0-3.0 GHz)

•   Edge  layer:  E =  {1, 2,..., E} represents  E  =  4 edge servers (4.0-6.0 GHz)

•   Cloud layer: C = {1, 2,..., C} represents  C = 2 cloud servers (8.0-12.0 GHz)

•   Task  set:  T  =  {1, 2,..., T} represents  T  =  40  tasks to  be  processed,  including  compute-intensive   (30%),

Fig. 1.   Device layer, edge layer, and cloud layer network architecture.

data-intensive   (25%),   real-time   sensitive   (25%),   and lightweight (20%)

B. Computation Model

1) Task Characteristics: Each task i ∈ T is  characterized by a five-tuple:

τi  = (Di , Ci ,Timax ,λi , AoImaxi)                  (1)

where:

•   Di  is the input data size (MB), Dresulti is the output data

size

•   Ci  is the computational complexity (CPU cycles/bit)

•   Timax  is the maximum tolerable delay (s)

•   λi  is the task update interval (s)

•   AoImaxi is the maximum acceptable age of information

(s)

2) Delay Model: The total task processing delay includes transmission delay, waiting delay, and computation delay:

Ti  = Titrans + Tiwait + Ticomp                             (2) Local execution delay only includes computation time:

(3)

where fi,d(i)  is the CPU frequency allocated to task i. The delay for offloading a task to an edge server:

where Rj,k  is the transmission rate from node j to k, calcu- lated using Shannon’s formula:

Queuing delay analysis: Considering edge  servers  adopt the M/M/1 queuing model, the average waiting time is:

(6)

where ρ = λe /µe  is the server utilization rate, λe  is the task arrival rate, and µe  is the service rate.

3) Energy Model: Computation energy consumption adopts a dynamic power model:

Eci,ojmp  = κj  · (fi,j )α · Di · Ci                             (7)

where  κj    ∈   [1.0, 3.5] × 10 −27   and  α   ∈  [2.2, 2.8]  are  set according to device type.

Transmission energy consumption: Eti,rja,nks  = Ptransj ·(Di + Dresulti)/Rj,k , where Ptransj ∈ [0.1, 0.5] W.

The total energy consumption of mobile devices:

(8)

4) AoI Model: To describe the freshness of information, we define the Age of Information (AoI) metric. For task i, its AoI at time t is defined as:

AoIi (t) = t − Ui (t)                        (9)

where Ui (t) is the time when task i last successfully completed an update.

Considering task processing delay and update interval, the average AoI of task i is:

i  = λi +                        (10)

This formula considers three parts: update interval λi , aver- age waiting time Ti /2, and additional aging due to processing delay Ti2 /(2λi ).

C. Optimization Problem Modeling

# 1) Decision Variables: We define binary decision variables

x = {xlocali, xedgei, xcloudi}i∈T, where:

•  xlocali indicates task i is executed locally on the device

•  xedgei indicates task i is offloaded to an edge server

•  xcloudi indicates task i is offloaded to a cloud server

Continuous decision variables f = {fi,j }i∈T,j∈D∪E∪C  repre- sent the allocated CPU frequency.

2) Objective Function: Based on practical application re- quirements and recent research trends, we use the weighted sum method to transform the tri-objective into a single objec- tive:

(11)

where:

•  Weight settings: wE  = 0.25, wT  = 0.35, wAoI  = 0.40 (prioritizing information timeliness)

•  Normalization   factors:    Enorm       =     maxi∈T Eilocal , Tnorm  = maxi∈T Ticloud , AoInorm  = maxi∈T AoImaxi

3) Constraints:  The   main  constraints  include:  (1)  Task

allocation  uniqueness:  xlocali  + xedgei  + xcloudi   =  1, ∀i;  (2)

Resource capacity: Σi xji ·fi,j  ≤ Fmaxj; (3) Delay constraint:

Ti   ≤ Timax ;  (4) AoI  constraint:  AoIi   ≤ AoImaxi;  (5)  QoS

constraint: 95% of tasks meet delay requirements.

4) Problem Complexity Analysis:  Theorem 1:  The  MEC task  offloading  and resource  allocation problem  considering AoI is NP-hard.

Proof :  Consider  a  special  instance:  only  one  edge  server (E  =  1),  all  tasks  have  the  same  data  size  D  and  com- putational  complexity  C,  and  transmission  delay  and  AoI constraints are ignored. In this case, the problem is equivalent to:

(12)

s.t.     , ∀i ∈ S,  fi  ≤ Fmax ,    fi  ≥ 0, ∀i ∈ S

where S is the set of tasks selected for edge execution, and

Fmax  is the total available CPU cycles of the server.

Define the ”weight” wi  = fi  and ”value” vi  for each task i

as:

(13)

Then  equation  (14)  transforms  into  the  classic  Knapsack problem of selecting items in a knapsack with capacity Fmax to maximize total value, which has been proven to be NP-hard. 	Furthermore, the complete model contains both binary vari- ablesx and continuous variables f, and the objective function includes  nonlinear  terms  (such  as  fiα ),  while  also  needing to  consider  the  temporal  dependencies  of  AoI  constraints. Therefore, it belongs to Mixed Integer Nonlinear Programming

## (MINLP), making the solution even more difficult. □

IV.  TLBO-HHO FUSION ALGORITHM DESIGN

A. Algorithm Motivation and Innovative Design Philosophy

TLBO achieves stable population evolution through teacher- student interaction mechanism, while HHO realizes dynamic search  by  simulating  hawk  swarm  hunting  behavior.   The proposed TLBO-HHO fusion algorithm adopts a two-phase in- tegrated architecture: (1) two-phase collaborative mechanism,

with  TLBO  guiding  global  exploration  and  HHO providing local  exploitation;  (2)  adaptive  parameter  coordination;  (3) AoI-aware constraint handling.

B. Solution Encoding and Initialization

Each  solution  Sk   is  encoded  as  a  mixed  vector:  Sk    =

[xk, fk, ak ],  where  xki  ∈ {0, 1, 2} represents  task  execution

location, fik  is CPU frequency, and aki is server allocation. We

adopt an intelligent initialization strategy with 30% heuristic and 70% random.

C. Improved TLBO and HHO Mechanisms

TLBO  adopts   an  adaptive  teaching  factor:  TF     =   1 + rand() · (1 + e−σpop /σmax ), and the enhanced teacher phase considers the influence of elite individuals. HHO introduces nonlinear  escape  energy:  E  =  2E0  · cos(πt/2Tmax ) · (1 + 0.1sin(4πt/Tmax )), avoiding premature convergence (see Al- gorithm 2).

D. Two-Phase Integration and AoI Optimization

Adaptive probability switching: pHHO  = pbase ·αconv ·βdiv , where pbase  = 0.7−0.6(t/Tmax ). AoI-aware fitness function:

(14)

Fitness(X) = wE  · E(X) + wT  · D(X) +wAoI  · AoI(X) + Penalty(X)

V.  EXPERIMENTS AND RESULTS ANALYSIS

A. Experimental Setup

1) MEC System Parameter Configuration: Based on actual MEC deployment scenarios and configurations of AWS Wave- length and Azure Edge Zones, the parameters are set as shown in Table I.

| Algorithm 1 Intelligent Initialization Strategy |  |
| --- | --- |
| Require: Population size P Ensure: Initialized population P 1: P ← ∅ |  |
| 2: nheur ← ⌊0.3 × P⌋ // Heuristic part |  |
| 3: | for i = 1 to nheur do |
| 4: | s ← HEURISTICINIT () |
| 5: | P ← P ∪ {s} |
| 6: | end for |
| 7: | nrand ← P − nheur // Random part |
| 8: | for i = 1 to nrand do |
| 9: | s ← RANDOMINIT () |
| 10: | P ← P ∪ {s} |
| 11: | end for |
| 12: | return P |

TABLE I

MEC SYSTEM PARAMETER CONFIGURATION

| Parameter | Value |
| --- | --- |
| Number of devices/edge/cloud servers Device/edge/cloud CPU frequency Transmission power range Bandwidth (device-edge/edge-cloud) | 20/4/2 0.2-3/4-6/8-12 GHz 0.1-0.5 W 10-50/100-1000 Mbps |

Algorithm 2 TLBO-HHO Two-Phase Integration Algorithm (with AoI Optimization)

Require: Population size N, Maximum iterations Tmax , Weights wE , wT ,wAoI Ensure: Optimal solution Xbest 1: P ← INTELLIGENTINIT (N); ElitePool ← ∅ 2: Evaluate all individual fitness (including AoI) 3: for t = 1 to Tmax do 4: pHHO ← ADAPTIVEPROB(t, P) 5: E ← ESCAPEENERGY(t); TF ← TEACHINGFACTOR(P) 6: for each Xi ∈ P do 7: if rand() < pHHO then 8: if ¦E¦ ≥ 1 then 9: Xnew ← HHO EXPLORE(Xi, Xbest ) 10: else 11: Xnew ← HHO EXPLOIT(Xi, Xbest ,E, J) 12: end if 13: else 14: Xteacher ← SELECTTEACHER(P,ElitePool) 15: Xnew ← TLBO TEACH(Xi, Xteacher ,TF ) 16: Xnew ← TLBO LEARN(Xnew , P) 17: end if 18: Xnew ← CONSTRAINTREPAIR(Xnew ) 19: fnew ← EVALUATE(Xnew ) + PENALTY(Xnew ) 20: if fnew < f(Xi ) then 21: Xi ← Xnew ; Update ElitePool 22: end if 23: end for 24: end for 25: return Xbest

2) Algorithm Parameter Settings:  The  optimal  parameter configuration was determined through orthogonal experiments and  grid  search.  Common  parameters:  population  size  50, maximum iterations 150, 5 independent runs, weight settings wE  = 0.25, wT  = 0.35, and wAoI  = 0.40. TLBO-HHO spe- cific parameters include: initial HHO probability pmax  = 0.7, final HHO probability pmin  = 0.1, elite pool ratio 10%.

B. Algorithm Convergence Analysis

Fig. 2 shows the convergence curve comparison of the four algorithms. TLBO-HHO drops rapidly in the initial phase (0- 25  iterations),  maintains  stable  improvement  in  the  middle phase,  and finally converges to 0.00102. In contrast, TLBO converges  stably  but  slowly  (0.00292),  GA  easily  falls  into local optima (0.00757), and GWO performs poorly (0.00367).

C. Performance Comparison Analysis

Fig.  3  shows  the performance  metrics  comparison  of the four algorithms. TLBO-HHO  achieves  an  average fitness of 0.00102, improving by 65.2%, 85.4%, and 74.6% compared to  TLBO,  GA,  and  GWO  respectively.  In  terms  of  energy

TABLE II

TASK TYPE DISTRIBUTION AND CHARACTERISTICS

| Task Type | Ratio(%) | Avg Load(G) | Update Int.(s) | Max AoI(s) |
| --- | --- | --- | --- | --- |
| Compute-intensive | 30 | 1.36 | 0.949 | 2.549 |
| Data-intensive | 25 | 3.23 | 1.214 | 4.114 |
| Real-time sensitive | 25 | 0.14 | 0.617 | 0.690 |
| Lightweight | 20 | 0.01 | 1.302 | 1.619 |

consumption, TLBO-HHO only consumes 3.19J, significantly lower than GA (31.14J). Although GA achieves the best delay (0.577s), its energy consumption is too high; GWO has the fewest AoI violations (4.6 times), but its overall performance is inferior to TLBO-HHO.

D. Task Allocation Strategy Analysis

Fig.  4   shows  the  task  allocation  location  comparison. TLBO-HHO adopts a highly localized strategy (97.5% local execution),  effectively  reducing  transmission  delay  and  en- ergy  consumption. In  contrast,  GA  overuses  edge resources (22.5%), and GWO has the highest cloud utilization (12.5%).

E. AoI Performance Analysis

Fig. 5 shows the AoI performance for different task types. TLBO-HHO performs best for real-time sensitive tasks, with 95% of AoI within  1 second. Through prioritized local exe- cution and reasonable resource allocation, the AoI of all task types remains within acceptable ranges.

VI.  CONCLUSION AND FUTURE WORK

A. Research Summary

This paper addresses the task offloading and resource alloca- tion optimization problem in mobile edge computing, propos- ing a two-phase integrated TLBO-HHO fusion algorithm that incorporates AoI into MEC task offloading optimization for the first time. Through theoretical analysis, algorithm design, and experimental validation, we draw the following conclu- sions:

Problem  modeling  contribution:  We   established  a  tri-  objective optimization model considering energy consumption, delay,  and  AoI,  with  weight  settings  of  0.25-0.35-0.40  to adapt  to  information  timeliness  requirements.  We  analyzed task waiting time through queuing theory and proved the NP- hard nature of the problem through theoretical proof, providing a theoretical basis for adopting metaheuristic algorithms. The model considers multi-dimensional constraints of actual MEC systems,  including resource  capacity, bandwidth  limitations, delay requirements, energy budget, and AoI constraints, mak- ing it closer to practical application scenarios.

Fig. 2.   Algorithm convergence curve comparison

Fig. 3.   Algorithm performance metrics comparison

Algorithm  design innovation:  The  proposed  two-phase  integrated TLBO-HHO algorithm achieves performance break- through through innovative fusion mechanisms. The algorithm uses TLBO’s teaching phase to guide global exploration and HHO’s hunting strategy to provide local exploitation, achiev- ing  dynamic  balance  between  exploration  and  exploitation through adaptive probability switching (0.7→ 0.1) and param- eter coordination strategies. In particular, the introduced AoI- aware fitness evaluation and constraint handling mechanisms

Fig. 4.   Algorithm task allocation comparison

Fig. 5.   Task type AoI performance analysis

enable the algorithm to effectively optimize information fresh- ness.

Significant performance improvement: In an experimental  scenario containing 20 devices, 4 edge servers, 2 cloud servers, and  40  tasks,  TLBO-HHO  achieved  fitness  value  improve- ments of 65.2%, 85.4%, and 74.6% compared to TLBO, GA, and  GWO respectively. The  algorithm performs particularly well in energy consumption, with an average energy consump- tion of only 3.19J, 89.8% lower than GA. Meanwhile, through a 97.5% local execution strategy, it effectively balances delay and AoI performance.

Practical application value: The  algorithm demonstrates  intelligent task allocation strategies that can adaptively adjust execution  locations  according  to  task  types.  For  real-time sensitive tasks, 95% of AoI is controlled within 1 second; for compute-intensive  tasks,  performance  is  guaranteed  through reasonable resource allocation. This differentiated processing strategy meets the requirements of actual MEC applications and has important practical value.

B. Research Limitations

Despite achieving significant results, this research still has the following limitations:

Static scenario assumption: The  current  model  assumes  that  task  characteristics  and  network  states  are  relatively stable, without fully considering user mobility, channel time- variability, and dynamic task arrivals. In actual MEC systems, these dynamic factors have important impacts on performance.

Centralized decision architecture: The algorithm adopts  centralized optimization, requiring global information collec- tion and computation. In large-scale distributed MEC scenar- ios, it may face issues such as high communication overhead, single point of failure, and limited scalability.

Insufficient security considerations: The data privacy pro-  tection,communication security, and malicious attack defense during task offloading are not fully considered.

Simplified AoI model:  The  current  AoI  model  assumes fixed task update intervals without considering random arrivals and non-periodic updates.

C. Future Research Directions

Based  on  this  research  and  existing  limitations,  future research directions include:

Distributed algorithm implementation: Research on dis-  tributed  versions  of  TLBO-HHO  to  reduce  communication overhead,  design  consensus-based  distributed  optimization frameworks, and achieve collaborative optimization between edge nodes.

Dynamic scenario extension: Consider user mobility and  dynamic task arrivals, design online optimization mechanisms, and research multi-timescale AoI optimization strategies.

Federated learning integration: Combine federated learn-  ing  to  protect  user  privacy,  design  privacy-preserving  dis- tributed task offloading mechanisms, and optimize AoI while ensuring data security.

Multi-objective optimization deepening:  Research  more  complex  multi-objective  optimization  frameworks,  consider other  objectives   such  as   service  quality  and  fairness,  and design generation and selection mechanisms for Pareto optimal solution sets.

Practical system deployment: Validate  algorithm perfor-  mance on real MEC testbeds, consider actual network proto- cols and system overhead, and develop optimization tools for industrial applications.

REFERENCES

[1]  Y.  Mao,  C.  You,  J.  Zhang,  K.  Huang,  and  K.  B.  Letaief,  “A  survey on  mobile  edge  computing:  The  communication  perspective,”  IEEE Commun. Surveys Tuts., vol. 19, no. 4, pp. 2322–2358, 4th Quart., 2017.

[2]  S. Sardellitti, G. Scutari, and S. Barbarossa, “Joint optimization of radio  and  computational  resources  for  multicell  mobile-edge  computing,” IEEE Trans. Signal Inf. Process. Netw., vol.  1, no.  2, pp.  89–103, Jun.  2015.

[3]  L.  Huang,  S.  Bi,  and  Y.  J.  A.  Zhang,  “Deep  reinforcement  learning for  online  computation  offloading  in  wireless  powered  mobile-edge computing  networks,”  IEEE Trans. Mobile Comput.,  vol.  19,  no.  11, pp. 2581–2593, Nov. 2020.

[4]  S.  Guo,  B.  Xiao,  Y.  Yang,  and  Y.  Yang,  “Energy-efficient  dynamic offloading and resource scheduling in mobile cloud computing,” in Proc. IEEE INFOCOM, San Francisco, CA, USA, Apr. 2016, pp. 1–9.

[5]  R.  V.  Rao,  V.  J.   Savsani,  and  D.  P.  Vakharia,   “Teaching–learning- based optimization: A novel method for constrained mechanical design optimization problems,”  Comput.-Aided Des.,  vol. 43, no. 3, pp. 303– 315, Mar. 2011.

[6]  A.  A.  Heidari,  S.  Mirjalili,  H.  Faris,  I.  Aljarah,  M.  Mafarja,  and  H. Chen, “Harris hawks optimization: Algorithm and applications,” Future Gener. Comput. Syst., vol. 97, pp.  849–872, Aug. 2019.

[7]  H.  Chen,  A.  A.  Heidari,  H.  Chen,  M.  Wang,  Z.  Pan,   and  A.  H. Gandomi, “Multi-population differential evolution-assisted Harris hawks optimization: Framework and case studies,” Future Gener. Comput. Syst., vol. 111, pp. 175–198, Oct. 2020.

[8]  B. Tu, F. Wang, Y. Huo, and X. Wang, “A hybrid algorithm of grey wolf optimizer and harris hawks optimization for solving global optimization problems with improved convergence performance,” Sci. Rep., vol.  13, no. 1, p. 22909, Dec. 2023.

[9]  C. Wang, F. R. Yu, C. Liang, Q. Chen, and L. Tang, “Joint computation offloading  and interference management in wireless cellular networks with mobile edge computing,” IEEE Trans. Veh. Technol., vol. 66, no. 8, pp. 7432–7445, Aug. 2017.

[10]  J. Liu, Y. Mao, J. Zhang,and K. B. Letaief, “Delay-optimal computation task scheduling for mobile-edge computing systems,” in Proc. IEEE Int. Symp. Inf. Theory (ISIT), Barcelona, Spain, Jul. 2016, pp.  1451–1455.

[11]  L. Wang, K. Wang, C. Pan, W. Xu, N. Aslam, and L. Hanzo, “Deep re- inforcement learning based dynamic trajectory control for UAV-assisted mobile edge computing,” IEEE Trans. Mobile Comput., vol. 21, no.  10, pp. 3536–3550, Oct. 2022.

[12]  Z. Li and Q. Zhu, “Genetic algorithm-based optimization of offloading and resource  allocation  in  mobile-edge  computing,” Information,  vol. 11, no. 2, p. 83, Feb. 2020.

[13]  L. Zang, X. Zhang, and B. Guo, “Federated deep reinforcement learn- ing  for  online  task  offloading  and  resource  allocation  in  WPC-MEC networks,” IEEE Access, vol. 10, pp. 9856–9867, 2022.

[14]  A. Ali,  S. A. A.  Shah, T. Al  Shloul, A. Khattak, A. Ahmad,  and A. Ali, “Multiobjective Harris Hawks optimization-based task scheduling in cloud-fog computing,” IEEE Internet Things J.,  vol.  11, no.  13, pp. 24334–24352, Jul. 2024.

[15]  M. A. Abd-Elmagid, N. Pappas, and H. S. Dhillon, “On the role of age of information in the Internet of Things,” IEEE Commun. Mag., vol. 57, no. 12, pp. 72–77, Dec. 2019.

[16]  S. Kaul, R. Yates, and M. Gruteser, “Real-time status: How often should one update?” in Proc. IEEE INFOCOM, Orlando, FL, USA, Mar. 2012, pp. 2731–2735.

[17]  L. Liu, J. Feng, X. Mu, Q. Pei, D. Lan, and M. Xiao, “Asynchronous deep reinforcement learning for collaborative task computing and on- demand resource allocation in vehicular edge computing,” IEEE Trans. Intell. Transp. Syst., vol. 24, no.  12, pp.  15513–15526, Dec. 2023.

[18]  Z.  Zhou,  P.  Liu,  J.  Feng,  Y.  Zhang,  S.  Mumtaz,  and  J.  Rodriguez, “Computation resource allocation and task assignment optimization in vehicular fog computing: A contract-matching approach,” IEEE Trans. Veh. Technol., vol. 68, no. 4, pp. 3113–3125, Apr. 2019.